



 EMBED Designer.Drawing.7  [image: image1.wmf]15/16"

(24mm)

Screw

hole

Center line

Backset 2" (50mm)

Strike 

location

1-3/16"

Door Face

Edge

Edge center line -->

Use Strike as

template, mark 

outline of Strike

and mortise 

1/16" deep for

flush mount

Screw

hole

Strike

H1

H2

15/16" W

(24mm)

x 1-1/8"

 Deep

For 1-3/4" thick door,  fold here and attach to the door at

desired height of Strike location.  

Mark the positions of 

holes

for  Face and  

Edge 

bores.  

For other door thickness, mark 

the holes on door Face 

side and 

draw a 

light line between 2

locators H1/H2. Center the 

Edge 

Template on 

door Edge for 

marks

, using the 'Horizontal Center Line L' and 

 H1/H2  line 

drawn as 

reference for the 

Height. If mortise for 

Strike 

has 

already 

been made 

in 

metal frame, 

position template 

using 

strike 

mortise as reference.

Mortise 1-5/8" W x

3" L x 1/2" Deep for

flush mount of the

Trim set 

TA10-PT

7/8" dia x 1/2"deep

Insert lock in mortise.

Mark outline 

of the 

lock front 

and mortise

7/32" 

(5.5mm) 

deep 

for 

flush mount.

Screw

hole

Screw

hole

Horizontal Strike Line U

TA10-P 

Trim

Lower Mounting 

screw hole for trim

1/4" dia drill through

Horizontal Strike Line L

Template for A578 Pocket lock case with TA10P  Privacy  or TA10T  T Turn Trim

Mortise minimum 3-1/8"(79mm) Deep

Use this template for

Privacy side or

Thumb Turn side

T Turn Hole/interior

Drill 1/2" deep for 

InteriorThumb Turn 

trim (TA10-T) only. 

1-5/8" dia x 1/2" deep

2"

4"

6"

2"

4"

6"

8"

10"


_1301739336.unknown

_1301739335.unknown

